The file entitled "Intra-event Data_SND.csv" includes intra-event concentration data for stormwater runoff and effluents of the GPT-biofilter/sand filter treatment train. These data represent the concentrations measured for the subsamples taken at an chronological order (subsample #no. and time in the file) during each event. The corresponding flow volume passed (in m3 and % of total volume) at a certain sampling point are also included in the file. A total of 11 rain events (coded by A to K) were covered between Sep. 2020 and Sep. 2021 on the sampling dates mentioned in the file. The codes of the sampling points are as follows:

SW: stormwater from the catchment (= GPT inflow)

GPT: gross pollutant trap outflow (= filter cells inflow)

SF: non-vegetated sand filter outflow

BFC: chalk-amended, vegetated biofilter outflow

BF: vegetated biofilter outflow

The subsamples have been analyzed for organic micropollutants and other conventional water quality parameters (42 parameters in total). The list of all pollutants with their abbreviations (as mentioned in CSV file) is as follows.

| Name of pollutant | Family | Parameter |
| --- | --- | --- |
| Naphthalene | PAHs | Nap |
| Acenaphthylene | PAHs | Acy |
| Acenaphthene | PAHs | Ace |
| Fluorene | PAHs | Flu |
| Phenanthrene | PAHs | Phen |
| Anthracene | PAHs | Anth |
| Fluoranthene | PAHs | Flth |
| Pyrene | PAHs | Pyr |
| Benz(a)anthracene | PAHs | BaA |
| Chrysene | PAHs | Chry |
| Benzo(b)fluoranthene | PAHs | BbF |
| Benzo(k)fluoranthene | PAHs | BkF |
| Benzo(a)pyrene | PAHs | BaP |
| Dibenz(a.h)anthracene | PAHs | DahA |
| Benzo(g.h.i)perylene | PAHs | Bper |
| Indeno(1.2.3.cd)pyrene | PAHs | InP |
| Sum of 16 PAHs | PAHs | PAH16 |
| Sum of carcinogenic PAHs | PAHs | PAHCar |
| Sum of non-carcinogenic PAHs | PAHs | PAHOth |
| Sum of LWM PAHs | PAHs | PAHL |
| Sum of MWM PAHs | PAHs | PAHM |
| Sum of HWM PAHs | PAHs | PAHH |
| 4-t-Octylphenol | Phenols | OP |
| 4-t-Octylphenol monoethoxylate | Phenols | OP1EO |
| 4-t-Octylphenol diethoxylate | Phenols | OP2EO |
| 4-t-Octylphenol triethoxylate | Phenols | OP3EO |
| Nonylphenol (mixture of isomers) | Phenols | NP |
| Nonylphenol monoethoxylate (mixture of isomers) | Phenols | NP1EO |
| Nonylphenol diethoxylate (mixture of isomers) | Phenols | NP2EO |
| Nonylphenol triethoxylate (mixture of isomers) | Phenols | NP3EO |
| C10 - C40 Fraction | Hydrocarbons | C10C40 |
| C10 - C12 Fraction | Hydrocarbons | C10C12 |
| C12 - C16 Fraction | Hydrocarbons | C12C16 |
| C16 - C35 Fraction | Hydrocarbons | C16C35 |
| C35 - C40 Fraction | Hydrocarbons | C35C40 |
| Bisphenol A | Phenols | BPA |
| Total Organic Carbon | Global | TOC |
| Suspended solids dried at 105°C | Global | TSS |
| Turbidity | Global | Turb |
| Electrical conductivity | Global | Cond |
| pH | Global | pH |
| Temperatur | Global | Temp |