The file entitled "StormwaterRunoffQualityData_SND.csv" contains event mean concentration (EMC) data on stormwater samples collected from 8 rain events (coded by A to H) during one year between September 2020 and September 2021. The samples have been analyzed for organic micropollutants and other conventional water quality parameters (42 parameters in total). EMCs have been mathematically generated by a Monte-Carlo simulation from measured concentrations in sub-samples collected during each event. The data present a distribution for each EMC with Q2.5, Q50, and Q97.5 percentiles and standard deviation from the mean. Besides, the number of detected and non-detected (censored) data of sub-samples is mentioned. The list of all pollutants with their abbreviations (as mentioned in CSV file) is as follows.

| Name of pollutant | Family | Parameter |
| --- | --- | --- |
| Naphthalene | PAHs | Nap |
| Acenaphthylene | PAHs | Acy |
| Acenaphthene | PAHs | Ace |
| Fluorene | PAHs | Flu |
| Phenanthrene | PAHs | Phen |
| Anthracene | PAHs | Anth |
| Fluoranthene | PAHs | Flth |
| Pyrene | PAHs | Pyr |
| Benz(a)anthracene | PAHs | BaA |
| Chrysene | PAHs | Chry |
| Benzo(b)fluoranthene | PAHs | BbF |
| Benzo(k)fluoranthene | PAHs | BkF |
| Benzo(a)pyrene | PAHs | BaP |
| Dibenz(a.h)anthracene | PAHs | DahA |
| Benzo(g.h.i)perylene | PAHs | Bper |
| Indeno(1.2.3.cd)pyrene | PAHs | InP |
| Sum of 16 PAHs | PAHs | PAH16 |
| Sum of carcinogenic PAHs | PAHs | PAHCar |
| Sum of non-carcinogenic PAHs | PAHs | PAHOth |
| Sum of LWM PAHs | PAHs | PAHL |
| Sum of MWM PAHs | PAHs | PAHM |
| Sum of HWM PAHs | PAHs | PAHH |
| 4-t-Octylphenol | Phenols | OP |
| 4-t-Octylphenol monoethoxylate | Phenols | OP1EO |
| 4-t-Octylphenol diethoxylate | Phenols | OP2EO |
| 4-t-Octylphenol triethoxylate | Phenols | OP3EO |
| Nonylphenol (mixture of isomers) | Phenols | NP |
| Nonylphenol monoethoxylate (mixture of isomers) | Phenols | NP1EO |
| Nonylphenol diethoxylate (mixture of isomers) | Phenols | NP2EO |
| Nonylphenol triethoxylate (mixture of isomers) | Phenols | NP3EO |
| C10 - C40 Fraction | Hydrocarbons | C10C40 |
| C10 - C12 Fraction | Hydrocarbons | C10C12 |
| C12 - C16 Fraction | Hydrocarbons | C12C16 |
| C16 - C35 Fraction | Hydrocarbons | C16C35 |
| C35 - C40 Fraction | Hydrocarbons | C35C40 |
| Bisphenol A | Phenols | BPA |
| Total Organic Carbon | Global | TOC |
| Suspended solids dried at 105°C | Global | TSS |
| Turbidity | Global | Turb |
| Electrical conductivity | Global | Cond |
| pH | Global | pH |
| Temperatur | Global | Temp |